| TABLE 155.5101.G.8.B: MINIMUM STACKING LANE DISTANCE FOR PARKING LOT ENTRANCE DRIVEWAYS1 |  |
| --- | --- |
| NUMBER OF OFF-STREET PARKING SPACES 1 2 | MINIMUM STACKING LANE DISTANCE (FT) 2 3, 4 |
| 1 - 49 | 25 |
| 50 - 249 | 50 |
| 250 - 499 | 100 |
| 500 or more | 100 + 15 ft for every additional 50 spaces beyond 500 |
| NOTES: 1. The requirements in this Table only apply to driveways that intersect with streets functionally classified as City of Pompano Beach streets. 2. Entrances into parking structures may be credited towards the stacking lane distance requirement provided the parking structure entrance is accessed from a development driveway and not a primary drive aisle. 3. Stacking lane distance is measured from the intersection of the inbound lanes of the driveway with the street right-of-way, along the centerline of the stacking lane, to its intersection with the centerline of the first entrance into a parking area or other internal intersecting driveway. 4. Divide the total parking spaces required by the number of driveways on the subject site to determine the number of off-street parking spaces to utilize for determining minimum stacking lane distance required. |  |